SeaLife ECOshot Sports camera (Item: SL321) Revised 1/23/2007

Confidential – Do not duplicate without the express written consent of Pioneer Research

- World’s First truly waterproof and shockproof Sports Camera

- 6.0 megapixel CCD for quality resolution and sharp pictures.

- Fast action – Quick start-up, shutter response and recycle time.

- scene modes

  - Auto, Sport, Night, Portrait, Landscape, backlight.

  - Sea mode for underwater exposure control and color correction

  - Ext Flash mode for integration with external flash to enhance color and brightness underwater.

- SPY mode – Take continuous pictures at set time intervals.

- Large 2” LCD for easy on-camera viewing

- Compact pocket size.

- Depth tested to 75 ft (23m)

- Fully rubber armored for sure grip

- Shockproof tested 6ft (2m)

- Video mode

- Instant focus from 2ft (60cm) to ∞

- 2 X AA battery reliable power supply

- 4X zoom - digital

Camera specifications

Image Sensor:	6.0 mega pixel CCD

Image Resolution:	2816 x 2112,  2272 x 1704, 1600 x 1200, 640x480

File format:	JPEG

Movie Resolution:	320x240 at 24 frames per second (fps)

File format:	Motion JPEG (.avi)

Lens:		Aperture (f-stop):    f/3.3

Focal Length:	6.95 mm (film equivalent = 42mm)

Focus type:	Fixed Focus

Focusing ranges:	Land:

Macro: 	60cm (2ft) to 1m (3ft)

Normal:	1m (3ft) to infinity

Underwater:

Macro: 	45cm (1.5ft) to 75cm (2.5 ft)

Normal:	75cm (2.5ft) to infinity

Shooting angle:	Land: 	46 degrees horizontal

Underwater:	35 degrees horizontal

Zoom:		4X progressive digital zoom

Playback:	1 - 4X zoom

Shutter speed:	Mechanical shutter

Land mode: 		1/10 to 1/1000 second

Sea mode:		1/60 to 1/1000 second

External Flash mode:	1/90 to 1/500 second

Shutter delay:	Less than 1 second

ISO:		Auto, 50, 100 and 200 speed

Flash:		Auto, Fill, Off, Red-Eye reduction,

(In External Flash mode:	Normal, Far and Macro Flash)

Exposure

programs:	Land & Sea® exposure programs.

Land modes: Auto, Sport, Night, Portrait, Landscape, Backlight,

underwater modes, External Flash mode, Sea mode

Exposure adjust:	Auto, Manual (0.5 EV step, -2 ~ +2 EV)

White Balance:	Auto, Manual, Daylight, Cloudy, Fluorescent, Tungsten

Self-Timer:	Yes: 5 second or 10 second delay

LCD		2” color TFT LCD

Memory;	Internal:	14MB NAND flash memory

External:	SD card up to 1gb (and MMC card support)

PC camera:	Yes 320X240 at 15fps

EXIF Format:	EXIF 2.2

USB:		Version 2.0

Pictbridge:	Yes, supported.

Systems

Requirements:	Windows: Pentium III 600 MHz or higher, Windows 2000 / ME / XP or later, 64MB RAM, 128MB hard disk space, CD-ROM drive, available USB port.

MAC: PowerPC G3/G4 or higher. OS 9.0 or later, 64MB RAM, 128MB hard disk space, CD-ROM drive, available USB port.

CD/Software:	Driver: USB Mass Storage for Windows 2000 / ME / XP or later; Mac OS 9.0 or later

Photo Express, Photo explorer (Windows Only)

International instruction manuals in English, German, French, Spanish, Italian,

Language

Support:	On-Screen Display: English, German, French, Spanish, Italian, and Chinese (T-C and S-C)

Connector:	USB connector,

Power:	Battery: 	2 AA

Battery life:	Approx. 100 pictures

Certification	CE, FCC, VCCI, BSMI

Weight	230 grams, 295 with batteries and SD card.

Dimensions	3.080”H x 4.730”W x 1.695”D

78.4mm x 120.1mm x 43.1mm

Environmental and Operational Specifications:

Depth:		Individually depth tested to 75’ (23 m)

Buoyancy:	Negative

Controls:	Control buttons for power, Shutter, menue, mode and 4-way directional buttons.

O-ring:	Silicone

Lens port:	Optical glass lens port

Lens ring adapts wide angle and telephoto lenses.

LCD:		Optical grade polycarbonate window

Tripod mount:	Standard ¼-20 tripod threads to adapt accessories.

Lanyard:	Padded lanyard with swiveled attachment clip

Hardware:	Marine grade Stainless Steel metal hardware.

Material:	Rubber overmolded polycarbonate housing.

Shock:	Drop tested to 6 feet / 2 meters.

Warranty:	1 year limited warranty.

Specifications subject to change without notice